Interface Control Document

of the LCGT Project

作成のための資料

| Subgroup Name | Data Acquisition System |
| --- | --- |
| Subgroup Leader | Daisuke Tatsumi National astronomical observatory of Japan |

| Doc version | Date | Description |
| --- | --- | --- |
| Rev. A-0 | 2009-05-15 | Kick-off draft |

APPROVAL AUTHORITIES

用語集 / Glossary

| GPS | Global Positioning system |
| --- | --- |
| DAQ | Data Acquisition System |
| ADC | Analog to Digital Converter |
| bps | bit per second (a unit to indicate data flow capability) |

1. 各サブシステムの簡単な定義 / Concise definition of the subsystem

LCGT建設完了までに各サブグループが責任を持って提供すべき機能と、それを実現するためのハードウェア、ソフトウェア。

| No. | 機能 / Function | ハードウェア・ソフトウェア / Hardware, software |
| --- | --- | --- |
| 1 | Main DAQ system | Analog Digital Converter, Signal conditioning filter (whitening filter) |
| 2 | Frame Maker | Computer: Real time operation system to storage data with GW FRAME FORMAT. |
| 3 | Detector Diagnosis | Computer: Online data analysis soft for detector diagnosis and trend-data archive |
| 4 | Environment Monitor | Computer: Environmental data monitoring soft, display for user interface. |
| 5 | Time Keeper and Global Positioning System | GPS, GPS signal transmission system, GPS time server with Rubidium atomic clock, sampling clock distribution system for ADC and DAC. |
| 6 | Raw Data Archive | Computer, tape archive system and data storage system: Archived data database and its human interface. |
| 7 | Pre Analysis Server | Computer: Data re-formatting for GW signal search |
| 8 | Data Distribution | Computer and high-speed network: Data distribution system for collaborator's demands |

2. 要求仕様 / Required specifications of the system

１で示した機能に対する要求仕様が記述されている LCGT design document version 2 での section 番号、関連 sub-system、Design document version 2 からの変更の有無と可能性。

| No. | 機能 / Function | Described section (**) | Related sub-system | 変更有り | 変更の可能性有り |
| --- | --- | --- | --- | --- | --- |
| 1 | Main DAQ system | 17.1 | Data analysis, Digital system |  |  |
| 2 | Frame Maker | 17.2 | Digital system |  |  |
| 3 | Detector Diagnosis | 17.3 | Digital system |  |  |
| 4 | Environment Monitor | 17.4 | Digital system |  |  |
| 5 | Time Keeper and Global Positioning System | 17.5 | Digital system |  |  |
| 6 | Raw Data Archive | 17.6 | Digital system |  |  |
| 7 | Pre Analysis Server | 17.7 | Data analysis |  |  |
| 8 | Data Distribution | 17.8 | Data analysis |  |  |

** Section in LCGT design document of version 2

LCGT design document version 2 の改訂版は、下記にまとめてあります。http://gw.icrr.u-tokyo.ac.jp:8888/JGWwiki/LCGT/subgroup/analysis/daq

これを更に「デジタル制御」との共通化などを検討した改訂版を年内に作成する予定。

3. インターフェース / Interface of the system

各サブシステムの詳細設計を行うにあたり、決定しなければならない仕様、他のサブシステムで決めて欲しい仕様、もしくは合意されている仕様

* 実際に建設が始まったら決まっていないと困ること、建設完了までに決まっていないといけないこと。

|  | 事項 / Item | 関連サブシステム / Related sub-system | 要求値 / Value | 合意済 / Confirmed |
| --- | --- | --- | --- | --- |
|  | 観測帯域 | Data analysis | 10 Hz - 10 kHz | by 神田 |
|  | ADC bit resolution | Data analysis | > 16 bits | by 神田 |
|  | Number of channels for GW search | Data analysis | 32 channels | by 神田 |
|  | Pre-processed data formats | Data analysis | TBD | not yet |
|  | Acceptable delay time to distribute observation data for collaborators | Data analysis | TBD | not yet |
|  | Data storage capability | Data analysis | TBD | not yet |
|  | Accuracy of time | Data analysis | < 1us | not yet |
|  | Computational capability for GW searches | Data analysis | TBD | not yet |
| * | Network capability | 坑内施設 | 10Gbps spec network switches Each 16 ports for 8 DAQ stations | not yet |
| * | Power consumption | 坑内施設 | TBD | not yet |
| * | Waist heat | 坑内施設 | TBD | not yet |
| * | Location and space | 坑内施設 | TBD | not yet |
|  | Installation schedule | ??? | TBD | not yet |
|  | Budget allocation | ??? | TBD | not yet |
|  | Man power allocation | ??? | TBD | not yet |

↑ 当該サブシステムが決めないといけないことには * 印を付けること。

0. サブシステムの現状 / Current status of the subgroup

a) Conceptual design (サブシステムの定義) は確定しましたか？		確定 / 未定

* 未定な部分がある場合、検討項目を簡単に記述して下さい。

* Conceptual design の変更に伴い、予想される影響（検討事項）と関連 sub-group。

b) Alternative design は残っていますか？		残っている / いない

* Alternative design の採用により生じると予想される影響（検討項目）と 関連sub-group。

デジタル制御 sub-group が新設されたため、データ取得のフロントエンド（ADC, signal conditioning filter) などの共通化もしくは境界領域の変更などの alternative design が生じた。

ただし、いまのところConceptual design には変更なし。

関連 sub-group: 干渉計部会デジタル制御 sub-group

c) 今年予算承認されたとした場合、年度内に決定すべき項目と関連 sub-group。

実験室内の配置、消費電力、廃熱量について坑内施設と打ち合わせる必要がある。