Interface Control Document of LCGT

| Subgroup Name | Vacuum System |
| --- | --- |
| Subgroup Leader | Yoshio Saito KEK-High Energy Accelerator Research Organization |
| Sub-leader | Ryutaro Takahashi National Astronomical Observatory of Japan |

| Doc version | Date | Description |
| --- | --- | --- |
| Rev. A-0 | 2009-05-28 | Kick-off draft |

APPROVAL AUTHORITIES

Concise definition of vacuum system

Beam tube

Two 3-km vacuum tubes are kept in 2x10-7Pa of vacuum pressure so as to reduce scattering-effects due to residual gas molecules.

The stainless-steel material SUS304L or SUS316 is among the most available for use as long tubes of 3km long and 1m in diameter. Since baking over the entire system of long tubes is not easy, further requiring a large electric power supply, some kinds of surface processing methods for reducing outgas are necessary to be applied to the unit tubes before installation in the tunnel. The ECB method is practically advantageous for large-chamber applications, because it can be performed without any large liquid reservoir or large coating system.

Baffles are characteristic of the beam tubes. The scattered light from the surface of tubes is blocked by the baffle. The surface of the baffle is treated with DLC coatings.

Chamber

Four kinds of vacuum chambers are used in LCGT. The size of each type is dependent in the structure of vibration isolation system for mirrors which are placed in the chamber; SAS + cryogenic mirror suspension in Type A chamber, SAS + mirror suspension in Type B chamber, and stack + mirror suspension in Type C chamber. The stainless-steel material SUS304 is among the most available for use as also large chambers of 1-2m in diameter.

Thirteen chambers are connected by the beam tubes with gate valves of DN1000 or DN400 each other.

Pumping system

From atmospheric pressure to several tens of Pa, a mechanical pump is first operated. A dry pump of the roots type equipped with multi-stages, recently being developed to reach a lower ultimate pressure of several Pa, is available.

To evacuate the system, 67 pumping units are placed at each chamber and every 100m along the 3-km tubes. After the tube pressure is reduced to be on the order of 10-5Pa by operating turbo-molecular pumps, Spatter ion pumps are considered to be most convenient for further pumping and keeping the vacuum.

Glossary

| SAS | Seismic Attenuation System |
| --- | --- |
| ECB | Electro-Chemical Buffing |
| DLC | Diamond-Like Carbon |
| TMP | Turbo Molecular Pump |
| SIP | Spatter Ion Pump |

Figure: Schematic drawing of the vacuum-system design.

Table: Specification of the vacuum system