Interface Control Document

of the LCGT Project

作成のための資料

| Subgroup Name | Scattered Light Control in IFO Support |
| --- | --- |
| Subgroup Leader | Shinji Miyoki ICRR, UT |

| Doc version | Date | Description |
| --- | --- | --- |
| Rev. A-0 | 2009-05-31 | Kick-off draft |

APPROVAL AUTHORITIES

用語集 / Glossary

| SLC | Scattered Light Control |
| --- | --- |
| DLC | Diamond Liked Carbon ( coating ) |
| NIP | Nickel Phosphorus ( coating ) |

1. 各サブシステムの簡単な定義 / Concise definition of Scattered Light Control

LCGT建設完了までに各サブグループが責任を持って提供すべき機能と、それを実現するための基本的対策。

| No. | 機能 / Function | 基本的対策 / Policy |
| --- | --- | --- |
| 1 | SLC for Main 3km Duct | - Diamond Liked Carbon (DLC) or NIP Coated Baffles |
| 2 | SLC for Radiation Shield Duct | - Diamond Liked Carbon (DLC) or NIP Coated Baffles - Radiation Protector coated by DLC or NIP around Mirrors in the inner shield. |
| 3 | SLC for Main IFO Optics | - Proper wedge - angle Low reflection low scattered loss AR coating - Optical dumping set for the high intensity AR reflected light - Optical dumping set for the high angle scattered light |
| 4 | SLC for Input Output Optics | - HR mirrors are desirable - Back side should be polished and AR coated - Proper wedge - Optical dumping set for the transmitted light |
| 5 | SLC for Vacuum Windows | - Proper edge - Double AR coated - Optical dumping set for the high intensity AR reflected light |
| 6 | SLC for Photo Detectors | - Housed in NIP coated box - Optical dumping set for the high intensity AR reflected light - Optical dumping set for the reflected light fro PDs |
| 7 | SLC for non-Linear Optics | - Optical dumping set for the high intensity AR reflected light - Brewster angle cutting |
| 8 | SLC for Beam Axes | - Iris - NIP coated meshed metal tube beam guide |

2. 要求仕様 / Required specifications Scattered Light Control

１で示した機能に対する要求仕様が記述されている LCGT design document version 2 での section 番号、関連 sub-system、Design document version 2 からの変更の有無と可能性。

| No. | 機能 / Function | Described section (**) | Required value | modified | to be modified |
| --- | --- | --- | --- | --- | --- |
| 1 | SLC for Main 3km Duct | 14.2 | Less than 1/10 of the targeted Sensitivity (2*10^-21 is designed) |  |  |
| 2 | SLC for Radiation Shield Duct | New | Less than 1/30 of the designed heat for one mirror (~ 300mW) Practical baffles design is proven in CLIK (Tomaru) Practical mirror shield is proven in CLIO (Uchiyama) |  |  |
| 3 | SLC for Main IFO Optics | New | Damp 10ppm loss from one mirror |  |  |
| 4 | SLC for Input Output Optics | New | Not fixed |  |  |
| 5 | SLC for Vacuum Windows | New | Not fixed |  |  |
| 6 | SLC for Photo Detectors | New | Not fixed |  |  |
| 7 | SLC for non-Linear Optics | New | Not fixed |  |  |
| 8 | SLC for Beam Axes | New | Not fixed |  |  |

** Section in LCGT Design Document

3. インターフェース / Interface of the Scattered Light Control

各サブシステムの詳細設計を行うにあたり、決定しなければならない仕様、他のサブシステムで決めて欲しい仕様、もしくは合意されている仕様

* 実際に建設が始まったら決まっていないと困ること、建設完了までに決まっていないといけないこと。

|  | 事項 / Item | 関連サブシステム / Related sub-system | 要求値 / Value | 合意済 / Confirmed |
| --- | --- | --- | --- | --- |
|  | SLC for Main 3km Duct | - Vacuum - SPI | - Outgas level | By Vacuum for DLC, not NIP |
| * | SLC for Radiation Shield Duct | - Vacuum - Cryostat - Refrigerator | - Refrigerator power to evacuate the heat load from 300K area. | By Cryostat system |
|  | SLC for Main IFO Optics | - Vacuum - ISC - Mirror Coating | - Position - Scattered light level, angle | Not yet |
| * | SLC for Input Output Optics | - Vacuum | - Housed in vacuum? | Not yet |
| * | SLC for Vacuum Windows |  |  | not yet |
| * | SLC for Photo Detectors | - Clean Environment - Sound Environment | - Clean air introduction - Box material | not yet |
| * | SLC for non-Linear Optics | - Input optics | - Scattered light level | not yet |
| * | SLC for Beam Axes | - Input Optics Design | - Layout | not yet |

↑ 当該サブシステムが決めないといけないことには * 印を付けること。

0. サブシステムの現状 / Current status of the Scattered Light Control

a) Conceptual design (サブシステムの定義) は確定しましたか？		確定 / 未定

* 未定な部分がある場合、検討項目を簡単に記述して下さい。

* Conceptual design の変更に伴い、予想される影響（検討事項）と関連 sub-group。

b) Alternative design は残っていますか？		残っている / いない

* Alternative design の採用により生じると予想される影響（検討項目）と 関連sub-group。

c) 今年予算承認されたとした場合、年度内に決定すべき項目と関連 sub-group。

輻射シールドにつけるバッフルのコート素材、位置、個数、について。

インナーシールド内の鏡輻射保護シールドの位置、コート素材について。

Optical dumping set for the high intensity AR reflected light　の方針について。

IFOのデザインにより決まった鏡の位置からのWedge角の逆算。